# METHOD OF STATEMENT

Date: 19th Aug 2021

Description of work

To hoist down 1 unit machine from lorry chassis to the ground, shift and position into factory premise at ground floor

Machine Spec

1 unit 686 CMM machine

Dim: 1150 x 1623 x 2638 mm

Wt: 730 kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

24/08/21

Time of Operation

12:00 pm

Location of Operation

#### 51 Joo Koon Circle

#### Omni Mold Ltd

Equipment Use

1 x lorry loader

Pallet truck

Webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated location

- Hoist down machine from lorry chassis

- Lower down machine to near ground level

- Insert pallet truck to machine base

- Transport machine into factory premise at ground floor

- Position machine at the designated location

- Unload machine from pallet truck and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175