# METHOD OF STATEMENT

Date: 01st Nov 2024

Description of work

To hoist down 5 cases of 3 unit machine from lorry chassis to the ground, uncrate, shift and position into factory premise at ground floor using pallet truck

Machine Spec

4 cases of 2 unit Form P350

2 case dim: 234 x 191 x 254 cm / Wt: 2, 130 kg each

2 case dim: 163 x 120 x 252 cm / Wt: 350 kg each

1 case of 1 WPT

Case dim: 221 x 185 x 256 cm / Wt: 1, 400 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

13/11/24

Time of Operation

09:45 am

Location of Operation

#### 51 Joo Koon Circle

#### Omni Mold Ltd

Equipment Use

2 x lorry loader

Pallet truck

Webbing sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deployment of  lorry loader at designated unloading area

- Set up crane with outrigger fully extended and resting of steel plate/timber mats as base plate

- Lorry crane to hoist down case from lorry chassis to the ground

- Uncrate case and put aside

- Rigger to attached sling to crane hook

- Secured sling to rigging point of machine

- Hoist and detach machine from base

- Swing crane and lower down machine to near ground level

- Insert pallet truck under machine bed

- Transport machine into factory premise at ground floor

- Position machine at the designated location

- Unload machine from pallet truck and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175