# METHOD OF STATEMENT

Date: 25th Jan 2024

Description of work

1) To hoist out 1 unit old compressor and air tank from compressor room and place aside

2) To unload and hoist down 1 unit new compressor and air tank from forwarder lorry, shift and position into compressor room

Spec

1 unit Compressor

1 unit Air Tank

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

Estimate:

19/02/24

Time of Operation

10:00 am

Location of Operation

#### 51 Loyang Dr

STEA Systems Pte Ltd

Equipment Use

1 x 633 lorry crane

Webbing sling

Shackle

Pallet truck/woodblock

Crowbar

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed

All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deployment of lorry crane at designated location for the hoisting work

- Crane operator to extract the crane boom to compressor room

- Shift out compressor to near crane

- Rigger to insert sling to crane hook

- Secure sling to rigging point of compressor

- Hoist up compressor 1ft above ground to check on balancing

- When well balance, place old compressor and tank aside

- To hoist new item to compressor room

- Lower down to near ground level

- Insert pallet truck under compressor base

- Shift and position to the exact location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175