# METHOD OF STATEMENT

Date: 14th Oct 2025

Description of work

To shift out 1 unit machine from production to loading area, hoist up and lay down machine, load onto low bed/lorry chassis and deliver to Eric Wong warehouse

Machine Spec

1 unit 110ton Press machine

Wt:10, 000 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

16/10/25

Time of Operation

09:30 am

Location of Operation

#### 55 Penjuru Rd

Sanmina-SCI Limited

Equipment Use

2 x 633 lorry loader

Hydraulic jack

Heavy duty rollers

Wire sling

Shackle

Metal plate

Obstacles

No obstacle on crane parking area

Barricade off operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Jack up machine to 4” high

- Insert rollers to machine base

- Shift out machine from production area

- Use of forklift to assist in pulling/pushing machine to loading bay

- Rigger to insert required lifting gears to crane hook and secured to rigging point of machine

- 1 x lorry loader to hoist up machine

- 1 x lorry loader to assist to lay down machine to horizontal position

- Hoist up machine and load onto low bed

- Secured machine with strapping belt/chain

- Transport out and deliver to Eric Wong warehouse

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175