# METHOD OF STATEMENT

Date: 02nd Aug 2021

Description of work

To unload 4 cases of 1 set machine from container, uncrate, upright machine, shift and transport into factory premise at ground floor

Machine Spec

Servo-hydraulic Universal Testing machine

Machine dim: 467 x 148 x 144 cm / Wt: 10, 200 kg

HPU: 107 x 157 x 152 cm / Wt: 694 kg

Grip: 94 x 52 x 65 cm / Wt: 258 kg

Access: 58 x 53 x 109 cm / Wt: 44 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

04/08/2021

Time of Operation

09:45 am

Location of Operation

#### 58 Sungei Kadut Loop

Admaterials Technologies Pte Ltd

Equipment Use

2 x 633 lorry loader -

Rollers/pallet truck

Wire/webbing sling

8.5 ton shackle

12 ton shackle

1 x platform (to lay out in front of door entrance for slope control)

Metal sheet/canvas (for painted floor protection)

Obstacles

No obstacle on crane parking area and route to machine positioning

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area (back to back with container)

- Pull out machine case from container onto lorry chassis

- Hoist down case to the ground

- Uncrate machine case and put aside

- Insert require sling to crane hook and secure to hoisting point of machine

- Using 2 x lorry loader to hoist and upright machine to vertical position

- Load machine onto platform

- Insert rollers under machine bed

- Shift and transport machine into production area at ground floor

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani / Kevin

Eric Wong Machinery Transportation Pte Ltd / Tel: 62824175