# METHOD OF STATEMENT

Date: 21st Feb 2022

Description of work

To unload 2 cases of 1 unit machine from lorry chassis, uncrate, shift and transport into factory premise at ground floor

Machine Spec

Brother Compact Machining Center

Case dim: 228 x 236 x 265 cm

Wt: 2, 230 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

21-24/02/22

Time of Operation

09:45 am

Location of Operation

#### 6 Second Chin Bee Rd

Dynacast (S) Pte Ltd

Equipment Use

1 x 633 lorry loader

Pallet truck

Wire/webbing sling

Obstacles

No obstacle on crane parking area and route to machine positioning

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Uncrate case

- Hoist up and detach machine from base

- Swing crane away from base and lower down machine to near ground level

- Insert pallet truck under machine bed

- Shift and transport machine into factory premise at ground floor

- Position machine at the designated location

- Unload machine from pallet truck and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175