# METHOD OF STATEMENT

Date: 17th Apr 2023

Description of work

To unload 1 unit machine from lorry chassis using forklift, shift and position into customer premise at ground floor

Machine Spec

1 unit machine

Dim: 2900 x 1700 x 2306 mm

Wt: 3, 000 kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

18/04/23

Time of Operation

Afternoon schedule

Location of Operation

#### 61 Seletar Aerospace View

Pratt & Whitney Services Pte Ltd

Equipment Use

2 x lorry crane

1 x 4.5ton forklift

Rollers/pallet truck

Hand jack/woodblock

Canvas/metal sheet (to lay out for floor protection)

Obstacles

No obstacle on unloading bay

Barricade operation area to prevent person who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Safety Aspect

Barricade off work area with safety cones, red/white tape and signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

M.O.M certified forklift operator

# JOB SCOPE

- Deploy lorry loader at designated unloading bay

- Using of forklift to fork down machine from lorry chassis

- Lower down machine to near ground level

- Transport machine to factory door entrance

- Lower down machine onto woodblock

- Insert rollers under machine bed

- Shift and transport machine into factory premise

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani / Kevin Wong

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175