# METHOD OF STATEMENT

Date: 05th Oct 2022

Description of work

To unload 1 unit machine from container, shift and transport into factory premise at ground floor

Machine Spec

1 unit Bruderer BSTA 30llHL

Dim: 210 x 115 x 265 cm

Wt: 5, 000 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

10-14/10/22

Time of Operation

09:30 am

Location of Operation

#### 65 Loyang Way

Bruderer Presses (Far East) Pte Ltd

Equipment Use

1 x 633 lorry loader

Rollers/pallet truck

Wire sling

Hand jack/woodblock

Obstacles

No obstacle on crane parking area and route to machine positioning

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area (back to back with container)

- Pull out base from container onto lorry chassis

- Loosen nut from machine

- Insert sling to crane hook and secure onto machine rigging point

- Hoist down machine from lorry chassis

- Swing machine away from base

- Lower down machine to near ground level

- Insert rollers/pallet truck under machine base

- Shift and transport machine into factory premise

- Position machine at the designated location

- Once job complete, team leader will instruct team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175