# METHOD OF STATEMENT

Date: 06th Aug 2025

Description of work

To shift out 1 unit machine from production to loading bay, hoist and load up onto lorry chassis and deliver to another location

Machine Spec

1 unit Bruderer BSTA 50

Dim: 290 x 200 x 260 cm

Wt: 8, 500 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

18/08/25

Time of Operation

10:00 am

Location of Operation

#### 65 Loyang Way

Bruderer Presses (Far East) Pte Ltd

Equipment Use

1 x 633 lorry crane

1 x platform (to lay out for machine pathway)

Rollers

Wire sling

Hydraulic/hand jack

Canvas/metal sheet (to lay out for floor protection)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed

All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry crane at designated location for the hoisting work

- Setting up of lorry crane with outrigger fully extended and resting on timber mats as base plate

- To shift out machine from production to loading bay

- Rigger to insert required sling to crane hook

- Secure sling to rigging point of machine

- Hoist up machine 1ft above ground to check on balancing

- When well balance, hoist up machine

- Load up machine onto lorry chassis

- Release all sling from machine rigging point

- Secured machine with strapping belt prior to moving out of premise

- Transport and deliver to new location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175