# METHOD OF STATEMENT

# Date: 27th Nov 2025

Description of work

To unload 1 case of 1 unit machine from lorry chassis using forklift, uncrate, shift and position into factory premise at ground floor

Machine Spec

1 units Hwacheon High Precision Conventional Lathe machine

Dim: 820 x 230 x 225 cm

Wt: 11, 000 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

05/12/25

Time of Operation

09:30 am

Location of Operation

#### 7 Benoi Rd

ST Engineering Marine Ltd

Equipment Use

1 x 633 lorry loader

1 x 16ton forklift

Rollers

Hydraulic jack

Woodblock

Canvas/metal plate (to lay out for floor protection)

Obstacles

No obstacle on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

M.O.M certified forklift operator

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Using 10ton forklift to fork down case from lorry chassis to the ground

- Uncrate case and put aside

- Forklift to fork up and detach machine from base

- Transport and unload machine at door entrance

- Using rollers to transport machine into factory premise

- Position machine at the designated location

- Unload machine from rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175