# METHOD OF STATEMENT

Date: 28th Mar 2023

Description of work

To shift out 1 unit container from production room to loading area, hoist up onto lorry chassis, transfer and unload at another location within NOV

Spec

1 unit Container Test Bay / Wt: 7, 500 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

28/03/23

Time of Operation

13:00 pm

Location of Operation

#### 7 Loyang Way

#### NOV Oilwell Varco

Equipment Use

1 x 633 PALFINGER lorry loader

Hand jack

Rollers

Wire sling

Obstacles

No obstacle on loading and crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Shift out container to outside factory

- Attached lifting gears to lorry crane hook and secure to rigging point of container to be lifted

- Hoist up container and load onto lorry chassis

- Transport to new location within NOV

- Hoist down container

- Position at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani	/ Kevin Wong

Tel: 62824175