# METHOD OF STATEMENT

Date: 03rd Jan 2023

Description of work

To shift out 5 unit machine and accessories from production room to loading area, hoist up and load onto wooden base for packing, after packing load onto low-bed/lorry chassis, deliver to customer appointed warehouse

Machine Spec

1 unit Doosan Puma 2500LSY CNC Lathe

Dim: 340 x 185 x 208 cm

Wt: 5, 400 kg

1 unit Doosan Puma 300LM CNC Lathe

Dim: 400 x 205 x 200 cm

Wt: 6, 700 kg

1 unit Doosan Puma 400l CNC Lathe

Dim: 510 x 215 x 230 cm

Wt: 9, 800 kg

1 unit Doosan Puma 480XLM CNC Lathe

Dim: 670 x 240 x 245 cm

Wt: 12, 500 kg

1 unit Doosan Puma MYNX 540/50 CNC Milling

Dim: 320 x 310 x 310 cm

Wt: 7, 000 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

03-06/01/23

Time of Operation

09:30 am

Location of Operation

#### 7 Loyang Way

#### NOV Oilwell Varco

Equipment Use

1 x 633 PALFINGER lorry loader

1 x forklift

Low loader

Hydraulic/hand jack

Heavy duty rollers

Pallet truck

Wire/webbing sling

Shackle

Obstacles

No obstacle on loading and crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Transport lorry loader into inside factory premise

- Attached lifting gears to lorry crane hook and secure to rigging point of machine to be lifted

- Hoist up machine and load onto lorry chassis

- Move out lorry with machine to packing area

- Hoist down machine onto wooden base

- Pack to form a case

- After packing, hoist/fork up case onto low-bed/lorry chassis

- Secured case with strapping belt prior to moving out of customer premise

- Transport and deliver to customer arranged warehouse

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani	/ Kevin Wong

Tel: 62824175