# METHOD OF STATEMENT

Date: 31st Oct 2022

Description of work

To shift out 3 unit machine and accessories from production room to loading area, hoist up and load onto lorry chassis

Machine Spec

1 unit Mazak J200

Dim: 440 x 270 x 140260 cm

Wt: 9, 300 kg

1 unit QTS 250M

Dim: 345 x 185 x 195 cm

Wt: 6, 200 kg

1 unit Everising Bandsaw

Dim: 280 x 210 x 205 cm

Wt: 4, 000 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

04/11/22   -   11/11/22

Time of Operation

09:40 am

Location of Operation

#### 7 Loyang Way

#### NOV Oilwell Varco

Equipment Use

1-2 x 633 PALFINGER lorry loader

1 x forklift

Hydraulic/hand jack

Heavy duty rollers

3/5 ton pallet truck

Wire/webbing sling

Shackle

Obstacles

No obstacle on loading and crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Jack up machine to 3” high using hydraulic/hand jack

- Insert rollers to machine bed

- Shift out machine from existing location

- Transport machine to designated hoisting area

- Attached lifting gears to lorry crane hook and secure to rigging point of machine to be lifted

- Hoist up machine and load onto lorry chassis

- Release all sling from rigging point of machine

- Secured case with strapping belt prior to moving out of customer premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175