# METHOD OF STATEMENT

Date: 03rd Jan 2024

Description of Work

To unload 1 set Welding system from 1 x 20ft OT container, shift and position into temporary shelter area

Machine Spec

4 cases of 1 set PWT Automates Welding System

Dim: 94 x 91 x 101.5” / Wt: 13, 000 lbs

Dim: 48 x 32 x 54” / Wt: 500 lbs

Dim: 48 x 32 x 54” / Wt: 500 lbs

Dim: 48 x 48 x 34” / Wt: 700 lbs

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

22/02/24

Time of Operation

10.00am

Location of Operation

#### 7 Seletar Aerospace Link

Windsor Airmotive Asia Pte Ltd

Equipment Use

1 x 633 lorry loader

Rollers/pallet truck

Wire/webbing sling

Shackle

Obstacles

No obstacles on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry crane at designated location (back to back with container)

- Using crane to pull out case from container

- Load case onto lorry chassis

- Transport case to unloading area

- Hoist down case from lorry chassis to the ground

- Uncrate 1 side of case and put aside

- Insert rollers/pallet truck onto case base

- Shift and transport into temporary shelter area

- Position case at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175