# METHOD OF STATEMENT

# Date: 20th May 2025

Description of work

To unload 2 units machine from 1 x 40ft FR container, uncrate, shift and position into factory premise at ground floor

Machine Spec

Auto TIG machine

Dim: 138” x 68” x 121” cm each

Wt: 5, 443 kg each

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

24/05/25

Time of Operation

10:00 am

Location of Operation

#### 7 Seletar Aerospace Link

Windsor Airmotive Asia Pte Ltd

Equipment Use

1 x 633 lorry loader

1 x 10ton forklift

Rollers/pallet truck

Hydraulic/hand jack

Woodblock

Canvas/metal plate (to lay out for floor protection)

Obstacles

No obstacle on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry crane operator and forklift driver will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety vest)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Unload 10ton forklift from low-bed via ramp down

- Using of 10ton forklift to fork down case to the ground

- Uncrate case and put aside

- Forklift to fork up and detach machine from base

- Transport and unload machine at door entrance

- Using rollers to transport machine into factory premise

- Position machine at the designated location

- Unload machine from rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175