# METHOD OF STATEMENT

Date: 16th Feb 2021

Description of work

To shift out 1 unit press machine from production room to loading area, hoist up and lay down machine, load onto wooden base, secured

Machine Spec

1 unit Press machine

Dim: 3.5m x 3.3m x 4.5m

Wt: 43, 000 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

TBA

Time of Operation

09:45 am

Location of Operation

#### 70 Joo Koon Circle

Nakano Metal Press (S) Pte Ltd

Equipment Use

1 x 250 ton mobile crane

1 x 50 ton lorry crane

Low-bed

1 x forklift

1 x scissor lift

Heavy duty rollers

Wire/webbing sling

Shackle

Metal plate/canvas (to lay out for floor protection)

Obstacles

No obstacle on crane parking area

Barricade off operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Assignment of traffic controller to movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Jack up machine to 4” high

- Insert rollers to machine base

- Shift out machine from production area

- Use of forklift to assist in pulling/pushing machine to loading bay

- Using of scissor lift for riggers to attached required lifting gears to crane hook and secured to lifting point of machine to be lifted

- 250ton mobile crane to hoist up machine

- Use 50ton lorry loader for assist to lay down machine to horizontal position

- Load machine onto wooden base and secured

- Hoist up machine and load onto low-bed

- Secured machine with strapping belt/chain prior to being transport out from premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175