# METHOD OF STATEMENT

Date: 13th Jan 2022

Description of work

To shift out 1 unit press machine from existing location to outside factory premise

Machine Spec

1 x 110ton Single Crank Press

Wt: 10, 000 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

21/01/22

Time of Operation

09:45 am

Location of Operation

#### 70 Joo Koon Circle

Nakano Metal Press (S) Pte Ltd

Equipment Use

Hydraulic jack

Heavy duty rollers

1 x forklift

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

M.O.M certified forklift operator will be involved in this operation

# JOB SCOPE

- Jack up machine to 4” high

- Insert rollers under machine base

- Use forklift to assist in pulling/pushing machine to outside factory premise

- Position machine at temporary designated location

- Jack up machine and insert hard wooden block under machine base

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175