# METHOD OF STATEMENT

Date: 01st Mar 2021

Description of work

To shift 1 unit press machine from one location to another temporary storage location

Machine Spec

1 unit Press machine

Dim: 3.5m x 3.3m x 4.5m

Wt: 43, 000 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

05/03/2021

Time of Operation

09:45 am

Location of Operation

#### 70 Joo Koon Circle

Nakano Metal Press (S) Pte Ltd

Equipment Use

1 x forklift

Heavy duty rollers

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Safety Aspect

Assignment of traffic controller to movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

M.O.M certified forklift operator will be involved in this operation

# JOB SCOPE

- Jack up machine to 4” high

- Insert rollers to machine base

- Use of forklift to assist in pulling/pushing machine to another location

- Jack up machine and insert hard wooden block for temporary storage

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175