# METHOD OF STATEMENT

Date: 18th May 2022

Description of work

To relocate 2 units machines from existing location to new location within building premise

Machine Spec

1 unit 150 Press machine/ Wt: 12, 000kgs

1 unit 80 Press machine / Wt: 6,000kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

20/05/22

Time of Operation

1pm

Location of Operation

#### 70 Joo Koon Circle

Nakano Metal Press (S) Pte Ltd

Equipment Use

Hand/hydraulic jack

Woodblock

Rollers/pallet truck

Metal sheet/canvas (to lay out for floor protection)

Obstacles

Ensure no obstacles around operation area and route to machine new location

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety helmet)

Job Scope

# Lift up machine to 3” high

# Insert pallet truck under machine base

# Shift out machine from existing location

# Transport to new location within premises

# Position machine at designated location

# Unload from pallet truck and insert levelling pad under machine bed

# Once job complete, team leader will instruct the team to perform proper housekeeping

# Job complete

# Best Regards

# Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175