# METHOD OF STATEMENT

Date: 01st Aug 2022

Description of Work

To unload 2 unit machine with accessories from low-bed/lorry chassis, shift and position into production area at ground floor

Machine Spec

1 unit Mazak VR1600W

Dim: 400 x 240 x 320 cm

Wt: 13, 000 kg

1 unit MPP50V4

Wt: 2, 000 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

18/08/22

Time of Operation

12:00 pm

Location of Operation

#### 71 Loyang Way

Kei & Wah Engineering Pte Ltd

Equipment Use

633 lorry loader

1 x low-bed

1 x forklift

Rollers

Pallet truck

Wire sling

Obstacles

No obstacles on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Reverse low-bed to near crane

- Insert sling to crane hook and secured onto rigging point of machine

- Hoist up machine from low-bed

- Swing crane and lower down machine to near ground level

- Insert rollers under machine bed

- Shift and transport machine into factory premise at ground floor

- Position machine at the designated location

- Unload machine form rollers and insert levelling pad under machine base

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175