# METHOD OF STATEMENT

Date: 14th Mar 2022

Description of work

To shift out 2 unit machine from production to loading area, hoist up onto lorry chassis and deliver to another location

Machine Spec

1 unit machine

Dim: 120 x 120 x 140 cm / Wt: 200 kg

1 unit machine

Dim: 100 x 150 x 150 cm Wt: 250 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

16/03/22 (Wednesday)

Time of Operation

09:45 am

Location of Operation

#### 8 Tractor Rd

Excelitas Technologies (S) Pte Ltd

Equipment Use

1 x lorry crane

Pallet truck

Wire/webbing sling

Obstacles

Ensure no obstacles around operation area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in place                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Insert pallet truck under machine base

- Shift out machine from production room to loading bay

- Insert sling to crane hook and secure onto machine hoisting point

- Lorry loader to hoist up machine

- Load machine onto lorry chassis

- Release all sling from crane hook

- Secured machine with strapping belt prior to moving out of customer premise

- Transport and deliver machine to new location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175