# METHOD OF STATEMENT

Date: 23rd Apr 2025

Description of Work

To hoist down cases from 2 x 40ft FR, 2 x OT container, uncrate, shift and position into production area at ground floor

Machine Spec

Fefer to attached packing list

6 cases of 1 unit Doosan NHM8000

5 cases of 1 unit Fronuis Cladding Rig

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

Estimate:

07/05/25 – 10/05/25

Time of Operation

09:30 am

Location of Operation

#### 80 Tuas West Dr

#### Dril-Quip Asia Pacific Pte Ltd

Equipment Use

1 x 100ton mobile crane

1 x 633 lorry loader

1 x scissor lift

Electric rollers/rollers

Wire/webbing sling

Shackle

Hydraulic jack

Woodblock

Metal plate/canvas (to lay out for floor protection)

Obstacles

No obstacles on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deployment of mobile crane at designated hoisting area

- Set up of crane with outrigger fully extended and resting on metal plate as base plate

- To hoist down case from FR container to the ground

- Uncrate case and put aside

- Riggers to insert sling to crane hook

- Secured sling to rigging point of machine

- Signalman to give signal to crane operator to hoist up and detach machine from base

- Swing machine away from base

- Lower down machine to near ground level

- Insert rollers under machine bed

- Shift and transport machine into factory premise at ground floor

- Position machine at the designated location

- Unload machine from rollers and insert leveling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175