# METHOD OF STATEMENT

Date: 18th Oct 2024

Description of work

To hoist up equipment to building roof top opening by using lorry crane with fly jib

Equipment Spec

1-1: DX Modular Air Handling unit / Dim: 1520 x 940 x 1440 mm / Wt: 208 kg

1-2: DX Modular Air Handling unit / Dim: 1520 x 710 x 1440 mm / Wt: 133 kg

1-3: DX Modular Air Handling unit / Dim: 1420 x 950 x 1620 mm / Wt: 310 kg

1-4: DX Modular Air Handling unit / Dim: 1420 x 860 x 1440 mm / Wt: 176 kg

1-5: DX Modular Air Handling unit / Dim: 1660 x 1420 x 1020 mm / Wt: 302 kg

1-6: DX Modular Air Handling unit / Dim: 560 x 810 x 660 mm / Wt: 213 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

23/10/24

Time of Operation

10:00 am

Location of Operation

#### 81 Pasir Ris Industrial Dr 1

#### Soitec

Equipment Use

1 x 75ton lorry crane with fly jib

2 x 633 lorry loader

Webbing sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

P.P.E Required

Safety Shoe

Safety Helmet

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed

Lifting machine and lifting gear with M.O.M certificate

Lifting crew shall be composed to the following but not limited by other trained personnel in respect to number of crew needed for the operation:

a. Lifting Supervisor

b. Lorry Crane Operator

c. Signalman

d. Riggerman

e. Skill Mover

# JOB SCOPE

- Deployment of lorry crane at designated hoisting area

- Set up crane with outriggers fully extended and resting on metal plate/timber mats as base plate

- Barricade off work area with safety cones, red/white tape and lifting in operation signage

- Rigger to insert sling to crane hook

- Secure sling to rigging point of equipment

- Hoist up 1ft above ground to check on balancing

- When well balanced, to hoist up load

- Extendt crane boom to exact distance of building rooftop

- Lower down crane boom until it reach the load location

- Position at the designated location

- To repeat process till all load have been hoist up

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175