# METHOD OF STATEMENT

Date: 11th Feb 2022

Description of work

To unload 3 cases of 1 unit machine from low bed to the ground, uncrate, shift and position into factory premise at ground floor

Machine Spec

D2-5AX machine

Case 1: 4000 x 2650 x 3870 mm / Wt: 10, 000 kg

Case 2: 3850 x 2630 x 3170 mm / Wt: 4, 500 kg

Case 3: 4300 x 2000 x 2100 mm / Wt: 1, 500 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

17/02/22

Time of Operation

09.45am

Location of Operation

#### 9 Joo Koon Crescent

Nutek Pte Ltd

Equipment Use

1 x 633 lorry loader

1 x low bed

1 x forklift

Heavy duty rollers

1 set wire sling

Shackle

Metal sheet/canvas (for epoxy floor protection)

Obstacles

No obstacle on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading bay

- Reverse low bed to near crane

- Hoist down machine case from low bed to the ground

- Uncrate case and put aside

- Attached required lifting gears onto crane hook and secured to rigging point of machine to be lifted

- Hoist up and detach machine from base

- Swing machine away from base

- Lower down machine to near ground level

- Insert rollers under machine bed

- Shift and transport machine into factory premise

- Position machine at the designated location

- Jack up machine, pull out rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175