# METHOD OF STATEMENT

Date: 18th Mar 2024

Description of Work

To hoist down 2 case from 1 x 40ft FR container to the ground, uncrate, shift and position machine into clean room

Machine Spec

Hermle C22 Machining Center

Case 1: 520 x 230 x 300 cm / Wt: 10, 035 kgs

Case 2: 362 x 140 x 145 cm / Wt: 907 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

Estimate:

22/03/24

Time of Operation

10:00 am

Location of Operation

#### 9 Tuas Dr 1

#### Fu Yu Corporation Ltd

Equipment Use

1 x 633 lorry loader

Rollers/pallet truck

Wire sling

Webbing sling

Shackle

Hydraulic jack

Woodblock

Metal plate/canvas (to lay out for floor epoxy protection)

Obstacles

No obstacles on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deployment of lorry crane at designated hoisting area

- Setting up of crane with outrigger fully extended

- Hoist down case from FR container to the ground

- Uncrate case and put aside

- Riggers to insert sling to crane hook

- Secured sling to rigging point of machine

- Hoist up and detach machine from base

- Swing machine away from base

- Lower down machine to near ground level

- Insert rollers under machine bed

- Shift and transport machine into factory premise at ground floor

- Position machine at the designated clean room

- Unload machine from rollers and insert leveling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175