# METHOD OF STATEMENT

Date: 10th June 2025

Description of Work

To hoist up Test Bench from ground level to 2nd floor by using lorry loader

Spec

20 units Test Bench

Dim: 240 x 120 x 6 cm each

Wt: 120 kg each

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

16/06/25 (Mon)

Time of Operation

09:30 am

Location of Operation

#### 91 Hillview Link, CMPB Regional Health Hub

CMPB HQ, CMPBFCC

Equipment Use

1 x 633 lorry loader

Pallet truck

Webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Barricade off work area with safety net, red white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated hoisting area for the lift

- Set up crane with outriggers fully extended and resting on timber mats as base plate

- Riggers to insert sling to crane hook

- Secured sling to rigging point of test bench

- Lorry loader to hoist up test bench to 2nd floor

- Lower down test bench to the ground

- Insert pallet truck onto test bench

- Shift into workshop premise

- Position at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175