# METHOD OF STATEMENT

Date: 15th Nov 2023

Description of work

To unload cases of 1 unit machine from 4 x 20ft FR, 1 x 40ft GP and 1 x 40ft HQ container  using forklift, uncrate, shift and position into customer premise at ground floor

Machine Spec

Vertical Machining Center / Variaxis C-600

Refer to packing list

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

23/11/23

Time of operation

09:45 am

Location of Operation

9A Seletar Aerospace Link

Coway Engineering & Marketing Pte Ltd

Equipment Use

2 x 633 lorry loader

1 x 16 ton forklift

Rollers/pallet truck

Hydraulic jack

Woodblock

Metal plate/canvas (to lay out for floor protection)

Obstacles

No obstacle around operation area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

M.O.M certified lifting supervisor, rigger, signalman, lorry loader and forklift operator will be involved in the operation

# JOB SCOPE

- Unload 16ton forklift from low-bed

- Using 16ton forklift to fork down cases from container

- Load cases to the ground

- Uncrate case

- Forklift to fork and detach machine from base

- Transport machine to loading bay

- Lower down machine to near ground level

- Insert roller under machine base

- Shift and transport machine into production area

- Position machine to the designated location at ground floor

- Unload machine from rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175