# METHOD OF STATEMENT

Date: 06th Jan 2025

Description of Work

To hoist down 14 cases of 2 unit machine from 5 x 40ft FR and 2 x 40ft GP container, uncrate, shift and position into production area at ground floor

Machine Spec

5 cases of YMS-9000 machine

Case 1: 632 x 350 x 362 cm / Wt: 28, 900 kg

Case 2: 519 x 200 x 340 cm / Wt: 5, 800 kg

Case 3: 300 x 210 x 135 cm / Wt: 1, 200 kg

Case 4: 453 x 237 x 218 cm / Wt: 2, 900 kg

Case 5: 290 x 213 x 192 cm / Wt: 2, 100 kg

9 cases of YMS-9001 machine

Case 1: 768 x 225 x 294 cm / Wt: 3, 700 kg

Case 2: 768 x 225 x 294 cm / Wt: 4, 500 kg

Case 3: 257 x 202 x 322 cm / Wt: 3, 110 kg

Case 4: 257 x 202 x 135 cm / Wt: 1, 450 kg

Case 5: 257 x 202 x 155 cm / Wt: 1, 600 kg

Case 6: 418 x 147 x 305 cm / Wt: 2, 100 kg

Case 7: 166 x 120 x 255 cm / Wt: 700 kg

Case 8: 450 x 220 x 150 cm / Wt: 1, 980 kg

Case 9: 450 x 220 x 140 cm / Wt: 2, 600 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

Estimate:

10/01/25

Time of Operation

09:45 am

Location of Operation

#### TSH Metal Tech Pte Ltd #01-51

Bulim Square, Blk 1 Bulim Lane 2

Equipment Use

1 x 100 ton mobile crane

633 lorry loader

Heavy duty rollers

Pallet truck

Wire/webbing sling

Shackle

Hydraulic jack

Woodblock

Metal plate/canvas (to lay out for floor protection)

Obstacles

No obstacles on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

PPE Required

Safety helmet

Safety shoe

Glove

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed

Lifting machine and lifting gear with M.O.M certificate

Lifting crew shall be composed to the following but not limited by other trained personnel in respect to number of crew needed for the operation:

a) Lifting Supervisor

b) Mobile Crane Operator

c) Lorry Loader Operator

d) Riggerman

e) Signalman

f) Skill Mover

# JOB SCOPE

- Deployment of mobile/lorry crane at designated hoisting area

- Set up of crane with outrigger fully extended and resting on metal plate/timber mats as base plate

- Reverse container to near crane

- To hoist down case from FR container to the ground

- Uncrate case and put aside

- Riggers to insert sling to crane hook

- Secured sling to rigging point of machine

- Signalman to give signal to crane operator to hoist up and detach machine from base

- Swing machine away from base

- Lower down machine to near ground level

- Insert rollers under machine bed

- Shift and transport machine into factory premise at ground floor

- Position machine at the designated location

- Unload machine from rollers and insert leveling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175