# METHOD OF STATEMENT

Date: 14th Nov 2025

Description of Work

To transport lorry loader with machine into tenant factory premise, hoist down machine from lorry chassis and position at the designated location

Machine Spec

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

Time of Operation

09:30 am

Location of Operation

#### TSH Metal Tech Pte Ltd #01-51

Bulim Square, Blk 1 Bulim Lane 2

Equipment Use

633 lorry loader

Rollers/pallet truck

Wire/webbing sling

Shackle

Hydraulic jack

Woodblock

Metal plate/canvas (to lay out for floor protection)

Obstacles

No obstacles on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

PPE Required

Safety helmet

Safety shoe

Glove

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed

Lifting machine and lifting gear with M.O.M certificate

Lifting crew shall be composed to the following but not limited by other trained personnel in respect to number of crew needed for the operation:

a) Lifting Supervisor

b) Lorry Loader Operator

c) Riggerman

d) Signalman

e) Skill Mover

# JOB SCOPE

- Transport of lorry loader with machine into tenant factory unit

- Set up of crane with outrigger fully extended and resting on metal plate/timber mats as base plate

- Riggers to insert sling to lorry crane hook

- Secured sling to rigging point of machine

- Signalman to give signal to crane operator to hoist up

- Swing crane and lower down machine to near ground level

- Position and unload machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175