# METHOD OF STATEMENT

Date: 04th Jan 2023

Description of work

To hoist down 1 unit old compressor from rooftop to ground floor and hoist up 1 unit new compressor to building rooftop

Compressor Spec

Dim: 125 x 85 x 170 cm

Wt: 200 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

14/01/23 (Saturday)

Time of Operation

09:00 am

Location of Operation

#### Blk 10 Kaki Bukit Ave 1

MMI Systems Pte Ltd

Equipment Use

1 x 50 ton mobile crane

Webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed

All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy mobile crane at designated location for the hoisting work

- Crane operator to extract the crane boom to building rooftop

- Insert sling to crane hook and secure onto rigging point of compressor

- Hoist up 1ft above ground to check on balancing

- When well balance, hoist down compressor from rooftop to the ground

- Unload compressor and place aside

- Release all sling from compressor rigging point

- To hoist up new compressor to building rooftop

- Unload at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175