# METHOD OF STATEMENT

Date: 15th Dec 2025

Description of Work

1) To hoist up 1 unit Bar Feeder to 5th floor through swing gate opening by using lorry loader

2) To unload 1 unit machine at loading bay, shift and position to 5th floor via cargo lift

Spec

Via hoisting

1 unit Bar Feeder

Dim: 3515 x 4585 x 1268 mm

Wt: 400 kg

Via cargo lift

1 unit BO126 VR machine

Wt: 2, 000 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

Estimate:

15/02/26 – 15/03/26

Time of Operation

10:00 am

Location of Operation

#### Blk 3014A Ubi Rd 1 #05-10

Anson Industries Technologies Pte Ltd

Equipment Use

1 x 633 lorry loader

Pallet truck

1 set webbing sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Barricade off work area with safety net, red white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated hoisting area for the lift

- Set up crane with outriggers fully extended and resting on steel plate/timber mats as base plate

- Rigger to insert webbing sling to lorry crane hook

- Secured sling onto Bar Feeder rigging point

- Lorry crane to hoist up Bar Feeder to 5th floor through swing gate opening

- Lower down Bar Feeder to the ground

- Insert rollers onto Bar Feeder stand

- Shift and position Bar Feeder into customer premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175