# METHOD OF STATEMENT

Date: 28th Feb 2024

Description of work

To hoist up 3 units machine to 2nd floor wall hacking opening by using platform and mobile crane / lorry crane

Machine Spec

2 unit MV500 Union Vertical Machining Center

Dim: 1.560 x 2.891 x 2.500 mm / Wt: 3.100 kg

1 unit MV700 Union Vertical Machining Center

Dim: 2.100 x 2.891 x 2.500 mm / Wt: 3.400 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

#### 06/04/24 (Saturday)

Time of Operation

13:00 pm

Location of Operation

#### Blk 3015 Ubi Rd 1 #02-226

Le-Plus Solutions Pte Ltd

Equipment Use

1 x 633 lorry crane

1 x mobile crane

2 x lorry loader

1 x platform

Pallet truck

1 set wire sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoe, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy mobile/lorry at designated hoisting area for the lift

- Hoist up platform to 2nd floor wall hacking opening

- Stabilize platform effectively and ensure platform 3ft metal protruding is fully lean and rested on the wall hacking opening slab

- Tied back platform to a competent point and secured using 1 x lorry crane

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- 1 x 50 ton mobile crane to hoist up machine to 2nd floor

- Lower down machine onto platform

- Insert pallet truck to machine base

- Use of manual pulley to slowly release and control the moving speed of machine

- Shift out machine from platform

- Position into customer premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175