# METHOD OF STATEMENT

Date: 08th Oct 2021

Description of work

To hoist up 1 unit work station to 3rd floor through swing gate opening, shift out from platform and position into customer premise

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

18/10/21 – 30/11/21

Time of Operation

16:00 PM

Location of Operation

#### Blk 3015A Ubi Rd 1 #03-02

Unique Precision Pte Ltd

Equipment Use

1 x 50 ton lorry crane

1 x lorry

1 x platform

Wire sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoe, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry crane at designated hoisting area for the lift

- Load work station onto platform

- Secured work station onto platform with strapping belt

- Hoist up work station together with platform to 3rd floor swing gate opening

- Stabilize platform effectively and ensure platform 3ft metal protruding is fully lean and rested on the swing gate opening slab

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Insert rollers/pallet truck onto equipment base

- Shift out from platform and position into customer premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175