# METHOD OF STATEMENT

Date: 18th Apr 2023

Description of work

To hoist up 2 unit EDM      machine to 3rd floor through swing gate opening by using platform and mobile/lorry crane

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

29/04/23   -   06/05/23 (Saturday job)

Time of Operation

09:30 am

Location of Operation

#### Blk 3015A Ubi Rd 1 #03-04

Unique Precision Pte Ltd

Equipment Use

1 x 50 ton mobile crane

1 x 633 lorry crane

Lorry loader

1 x platform

1 set wire sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry/mobile crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoe, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy mobile/lorry at designated hoisting area for the lift

- Hoist up platform to 3rd floor swing opening

- Stabilize platform effectively and ensure platform 3ft metal protruding is fully lean and rested on the swing gate opening slab

- Tied back platform to a competent point and secured using 1 x 50 ton lorry crane

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- 1 x 50 ton mobile crane to hoist up machine to 3rd floor

- Lower down machine onto platform

- Insert rollers to machine base

- Use of manual pulley to slowly release and control the moving speed of machine

- Shift out machine from platform

- Position into customer premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175