# METHOD OF STATEMENT

Date: 14th Mar 2022

Description of work

To hoist up 2 unit machine to 3rd floor through wall hacking opening, shift out from platform and position into customer premise

Machine Spec

2 unit VCN 530

Dim: 3000 x 3000 x 2900 mm

Wt: 6, 800 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

02/04/22

Time of Operation

09:45 am

Location of Operation

#### Blk 4010 A.M.K Ave 10 #03-05

Song Tat Precisions Pte Ltd

Equipment Use

1 x 50 ton mobile crane

1 x 50 ton lorry crane

1 x lorry crane

1 x low bed

1 x platform

Wire sling/shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry/mobile crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoe, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy mobile/lorry crane at designated hoisting area for the lift

- Hoist up platform to 3rd floor wall hacking opening

- Stabilize platform effectively and ensure platform 3ft metal protruding is fully lean and rested on the swing gate opening slab

- Tied back platform to a competent point and secured using 1 x 50 ton lorry crane

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- 1 x 50 ton mobile crane to hoist up machine to 3rd floor

- Lower down machine onto platform

- Insert rollers to machine base

- Shift out machine from platform and position into customer premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Remark:

Hacking work by Wofook Renovation & Contractor

To hack 1 layer internal parapet wall and make good upon job completion

Dismantle sun breaker and make good upon job completion

Dismantle door frame, hack L shape wall, dismantle window frame and make good upon job completion

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175