# METHOD OF STATEMENT

Date: 25th Nov 2025

Description of work

To fork down 1 unit machine from lorry chassis using forklift, unload at loading area, shift and position to 2nd floor via cargo lift

Machine Spec

1 unit QTE-200

Wt: 3, 700 kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

05/12/25

Time of Operation

10:00 am

Location of Operation

#### Blk 5 A.M.K Pk 2A #02-15

Yi Fong Precision Pte Ltd

Equipment Use

1 x lorry crane

1 x 4.5ton forklift

Rollers/pallet truck

Hand jack

Woodblock

Obstacles

No obstacle on unloading bay

Barricade operation area to prevent person who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

M.O.M certified forklift operator

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Using 4.5ton forklift to fork down machine from lorry chassis

- Transport machine to unloading area

- Lower down machine onto woodblock

- Insert rollers under machine bed

- Shift and transport to 2nd floor via cargo lift

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175