# METHOD OF STATEMENT

Date: 23rd Oct 2025

Description of work

Using lorry crane to hoist down 1 unit from 3rd floor through swing gate opening

Machine Spec

1 unit 5HP Milling machine

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

Estimate:

15/11/25

Time of Operation

09:30 am

Location of Operation

#### Blk 512 Chai Chee Lane #03-03

Metalcraft Precision Industries

Equipment Use

1 x 633 lorry loader

Wire sling/shackle

Pallet truck

Crowbar/woodblock

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

P.P.E Required

Safety shoe

Safety helmet

Safety Harness

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed

Lifting machine and lifting gear with M.O.M certificate

Lifting crew shall be composed to the following but not limited by other trained personnel in respect to number of crew needed for the operation:

a. Lifting Supervisor

b. Lorry Loader Operator

c. Signalman

d. Riggerman

e. Skill Mover

# JOB SCOPE

- Deploy lorry crane at designated location for the hoisting work

- Setting up of lorry crane with outrigger fully extended

- Crane operator to extract the crane boom to 3rd floor swing gate opening

- Rigger to insert required sling to crane hook

- Secure sling to rigging point of machine

- Hoist up machine 1ft above ground to check on balancing

- When well balance, hoist down machine slowly the ground

- Load up machine onto lorry chassis

- Secured machine with strapping belt prior to moving out of premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175