# METHOD OF STATEMENT

Date: 31st May 2023

Description of work

To unload 2 unit machine from lorry chassis using forklift, shift and position into customer premise at ground floor

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

06/06/23

Time of Operation

09:30 am

Location of Operation

#### Blk 7 Kallang Place #01-08

Tsugami Universal Pte Ltd

Equipment Use

1-2 x lorry crane

1 x 4.5ton forklift

Rollers/pallet truck

Obstacles

No obstacle on unloading bay

Barricade operation area to prevent person who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Safety Aspect

Barricade off work area with safety cones, red/white tape and signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

M.O.M certified forklift operator

# JOB SCOPE

- Deploy lorry loader at designated unloading bay

- Using of forklift to fork down machine from lorry chassis

- Lower down machine to near ground level

- Transport machine to factory door entrance

- Lower down machine onto woodblock

- Insert pallet truck under machine bed

- Shift and transport machine into factory premise at ground floor

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani / Kevin Wong

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175