# METHOD OF STATEMENT

Date: 05th July 2023

Description of work

To shift out 24 unit machines and accessories from back lane entrance of tenant premise, hoist and load up onto lorry chassis

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

Estimate:

07-08/07/23

Time of Operation

09:45 am

Location of Operation

#### Blk 998 Toa Payoh North #01-21

Apex Technology

Equipment Use

4-5 x 633 lorry loader

1 x platform - to lay out for machine pathway

Rollers

Pallet truck

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry crane at designated loading area for the hoisting (back lane of tenant premise)

- To hoist down and lay out platform in front of tenant door for machine pathway

- Shift out machine from production into platform

- Insert sling to crane hook and secured to rigging point of machine

- Hoist up machine

- Swing crane and load up machine onto lorry chassis

- Release all sling from crane hook

- Secured machine with strapping belt prior to moving out of tenant premise

- Deliver and deliver machine to other factory

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175