# METHOD OF STATEMENT

Date: 30th May 2025

Description of work

Lorry crane to hoist down 6 set jig crane after engineer dismantle, load onto lorry chassis and delivery to UMS factory

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

04/06/25 to 05/06/25

Time of Operation

10:00 am

Location of Operation

#### 16 Seletar Aerospace Crescent

#### Jep Precision Engieering Pte Ltd

Equipment Use

1 x 633 lorry loader

Webbing sling

Shackle

Pallet truck

Canvas/metal plate (to lay out for floor protection)

Obstacles

No obstacle on loading and crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Transport lorry crane into inside factory premise

- Deploy lorry loader at designated hoisting area

- Rigger to attached sling to crane hook

- Secure sling to rigging point of jig crane

- After engineer dismantle, hoist down jig crane and load onto lorry chassis

- Release all sling from rigging point of jig crane

- Secured with strapping belt prior to moving out of customer premise

- Transport and deliver to UMS factory

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175