# METHOD OF STATEMENT

Date: 02nd July 2025

Description of work

Using lorry loader to hoist down 1 bundle of plywood from lorry chassis to the ground

Spec

1 bundle of plywood

Wt: 2, 000 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

08/07/25

Time of Operation

09:00 am

Location of Operation

Kranji Camp 3

Equipment Use

1 x lorry loader

Webbing sling

Obstacles

No obstacle on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Set up crane with outriggers fully extended and resting on timber mats as base plate

- Rigger to insert webbing sling under pallet

- Secured sling to crane hook

- Hoist up plywood from lorry chassis

- Swing crane and lower down plywood to the ground

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 6282417