# METHOD OF STATEMENT

Date: 22nd Apr 2021

Description of work

To unload 9 cases of 1 unit machine from 1 x 40ft FR, 1 x 20ft FR and 2 x 40ft GP container, uncrate, shift and position into factory premise at ground floor

Machine Spec

Mazak Laser FMS Extensible Cell 3015

Case 1: 4720 x 2570 x 2630 mm / Wt: 6, 930 kg

Case 2: 4220 x 2370 x 1880 mm / Wt: 4, 380 kg

Case 3: 4040 x 2560 x 3160 mm / Wt: 3, 620 kg

Case 4: 3670 x 2040 x 1800 mm / Wt: 1, 100 kg

Case 5: 3220 x 2150 x 1210 mm / Wt: 1, 180 kg

Case 6: 2900 x 1560 x 1970 mm / Wt: 1, 350 kg

Case 7: 3440 x 2170 x 1610 mm / Wt: 1, 530 kg

Case 8: 8500 x 2210 x 1480 mm / Wt: 3, 720 kg

Case 9: 2420 x 2260 x 1610 mm / Wt: 1, 330 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

Est: 30/04/2021

Time of Operation

09:45 am

Location of Operation

#### 18 Joo Koon Circle

Yamazaki Mazak Singapore Pte Ltd

Equipment Use

1-2 x 633 lorry loader

1 x forklift

Wire sling

Rollers/pallet truck

Obstacles

No obstacle on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated location for unloading of cases

- Insert sling under machine case

- Lorry loader to hoist down case from FR container

- Load case onto the ground

- Uncrate case

- Hoist up and detach machine from base

- Lower down machine near door entrance

- Insert rollers/pallet truck under machine base

- Shift and transport machine into factory premise

- Position at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175