# METHOD OF STATEMENT

Date: 01st Sep 2021

Description of work

To uncrate machine cases, shift and position machine into factory premise at ground floor

Machine Spec

Laser Processing machine / OPTIREX 3015DDL

Case 1: 670 x 307 x 251 cm / Wt: 11, 325 kg

Case 2: 415 x 230 x 240 cm / Wt: 1, 029 kg

Case 3: 400 x 230 x 155 cm / Wt: 3, 340 kg

Case 4: 225 x 150 x 240 cm / Wt: 1, 034 kg

Case 5: 220 x 120 x 220 cm / Wt: 510 kg

Case 6: 200 x 200 x 250 cm / Wt: 909 kg

Case 7: 225 x 120 x 120 cm / Wt: 136 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

02/09/2021

Time of Operation

09:45 am

Location of Operation

#### 18 Joo Koon Circle

Yamazaki Mazak Singapore Pte Ltd

Equipment Use

1 x 633 lorry loader

Wire/webbing sling

Rollers

Pallet truck

1 x forklift

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated location for the hoisting

- Uncrate machine case

- Hoist up and detach machine from base

- Swing crane away from base

- Lower down machine to near ground level

- Insert rollers/pallet truck under machine base

- Shift and transport machine into factory premise

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175