# METHOD OF STATEMENT

Date: 28th Dec 2020

Description of work

To unload 7 cases of 1 unit machine from 1 x 40ft FR and 1 x 40ft HQ and container, shift and position into factory premise at ground floor without uncrating

Machine Spec

Laser Processing Machine – M/OPTIREX 3015 DDL

Case 1: 670 x 307 x 251 cm / Wt: 12, 800 kg

Case 2: 415 x 230 x 240 cm / Wt: 1, 459 kg

Case 3: 400 x 230 x 155 cm / Wt: 3, 530 kg

Case 4: 225 x 150 x 240 cm / Wt: 1, 262 kg

Case 5: 220 x 120 x 220 cm / Wt: 660 kg

Case 6: 200 x 200 x 250 cm / Wt: 1, 105 kg

Case 7: 225 x 120 x 120 cm / Wt: 255 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

Est: 04-05/01/2021

Time of Operation

09:45 am

Location of Operation

#### 18 Joo Koon Circle

Yamazaki Mazak Singapore Pte Ltd

Equipment Use

1-2 x 633 lorry loader

Wire/webbing sling

Rollers

Pallet truck

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated location for the hoisting

- Hoist down case from FR container to the ground

- Insert rollers/pallet truck under machine case

- Shift and transport case into factory premise

- Position at temporary location without uncrating

- Once job complete, team leader will instruct the team to perform proper housekeeping

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175