# METHOD OF STATEMENT

Date: 14th Oct 2020

Description of work

1) To relocate 1 unit Optical Table Measuring from existing lab at B1 to another lab at B2 via cargo lift

2) Deploy lorry at unloading area, hoist down forklift from lorry chassis, to shift out 1 unit Xray CT System Measuring from lab to loading bay, using forklift to fork up onto lorry chassis and deliver to another location

Machine Spec

1 unit Optical Table Measuring

Dim: 2.4 x 0.9 x 1.0 m

1 unit Xray CT System Measuring

Dim: 1.71 x 1.43 x 2.08 m

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

30/10/2020

Time of Operation

09:45 am

Location of Operation

Singapore Institute of Mfg Technology

Collection: NTU Valley Block

Deliver: 4 Fusionopolis Way, FP2 Kinesis Building #B1-26

Equipment Use

1 x 633 lorry loader

1 x forklift

Crowbar

Woodblock

Rollers/pallet truck

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent person who are not involved enter unintentionally

Environment

No lifting operation will be carry out in heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervised by M.O.M certified lifting supervisor, rigger and signalman

Job Scope

# (1)

# Insert pallet truck under table stand

# Shift out from existing lab at B1

# Transport to new lab at B2 via cargo lift

# Position at the new designated location

# (2)

# Insert pallet truck under machine base

# Shift out from lab to loading bay

# Using forklift to fork up machine

# Load machine onto lorry chassis

# Secured machine with strapping belt

# Transport machine to another location

# Unload machine at loading bay

# Transport machine into lab at B1-26

# Position machine at the designated location

# Job complete

# Best Regards

# Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175