# METHOD OF STATEMENT

Date: 22nd Apr 2021

Description of Work

To unload 2 cases of 1 unit machine from 1 x 20ft FR and 1 x 20ft GP container, uncrate, shift and position into customer premise at ground floor

Machine Spec

1 unit Sirius UM+ machine

Case 1: 300 x 265 x 330 cm

Wt: 8, 600 kgs

Case 2: 406 x 200 x 227 cm

Wt: 1, 550 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

Est: 27/04/21

Time of Operation

10:00 am

Location of Operation

#### 5 Tuas Dr 1

Fu Yu Corporation Pte Ltd

Equipment Use

1 x 633 lorry loader

Rollers/pallet truck

Wire/webbing sling

Shackle

Obstacles

No obstacles on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated location for the lifting

- Hoist down case from FR container to the ground

- Uncrate case

- Secured sling to machine hoisting point

- Hoist up and detach machine from base

- Swing machine to unloading area

- Lower down machine to near ground level

- Insert rollers to machine base

- Shift and transport machine into factory premise

- Position machine at the designated location

- Unload machine from rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175