# METHOD OF STATEMENT

Date: 03rd July 2017

Description of Work

To unload 1 unit machine from 1 x 20ft GP container, uncrate, shift and position into tool room

Machine Spec

1 unit Waida SPG-W CNC Profile Grinder

Dim: 1995 x 1615 x 2120 mm

Wt: 4, 500 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

11/07/2017   -   10:00 am

Location of Operation

#### 3 Sixth Lok Yang Rd

Singapore Kobe Pte Ltd

Equipment Use

1 x lorry crane

Rollers/pallet truck

Wire/webbing sling

Metal plate (for floor protection)

Obstacles

No obstacles on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman, lorry loader operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, etc..)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lorry crane to park at designated location (back to back with container)

- Pull out machine case from container onto lorry chassis

- Hoist down case from lorry chassis to the ground

- Uncrate machine case

- Hoist up and detach machine from base

- Swing crane to loading area

- Insert pallet truck/rollers to machine base for the move

- Shift and position machine into tool room

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175