# METHOD OF STATEMENT

Date: 19th June 2018

Description of Work

To unload 3 cases of 1 unit machine from lorry chassis, uncrate, shift and position into premise at ground floor

Machine Spec

GLS-150GL Amada CNC Precision Profile Grinder

Dim: 270 x 225 x 225 cm   /   Wt: 3, 950 kgs

Dim: 165 x 110 x 110 cm   /   Wt: 110 kgs

Dim: 142 x 104 x 106 cm   /   Wt: 50 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

27/06/2018   -   09:45 am

Location of Operation

#### 9 Sixth Lok Yang Rd

Singapore Kobe Pte Ltd

Equipment Use

1 x lorry crane

Rollers/pallet truck

Wire/webbing sling

Metal plate/canvas (for floor protection)

Obstacles

No obstacles on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, etc..)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Hoist down cases from lorry chassis to the ground

- Uncrate machine case

- Hoist up and detach machine from base

- Swing crane to unloading area

- Insert pallet truck/rollers to machine base for the move

- Shift and transport machine into premise at ground floor

- Position machine to the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175