# METHOD OF STATEMENT

Date: 08th Feb 2018

Description of Work

To unload 1 unit machine from lorry chassis, uncrate, shift and position into production at ground floor

Machine Spec

1 unit Vertical Milling machine

Dim: 1.83 x 1.83 x 2.21 m

Wt: 1, 315 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

09/02/2018   -   10:00 am

Location of Operation

#### No 2 Pioneer Sector 3

Ge Repair Solutions (S) Pte Ltd

Equipment Use

1 x lorry crane

Pallet truck

Wire sling

Shackle

Obstacles

No obstacles on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, etc..)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lorry crane to park at designated location

- Uncrate case

- Hoist and detach machine from base

- Swing crane to loading area

- Insert pallet truck to machine base

- Shift machine into production and position at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Remark:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175