# METHOD OF STATEMENT

Date: 09th Oct 2017

Description of Work

To unload 1 unit machine from 1 x 40ft HQ container, uncrate, shift and position into production at ground floor

Machine Spec

1 unit Global Performance 12.15.10

Case Dim: 3418 x 2178 x 2549

Wt: 4, 667 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

12-13/10/2017   -   10:00 am

Location of Operation

#### No 2 Pioneer Sector 3

Ge Repair Solutions (S) Pte Ltd

Equipment Use

1 x 35-45 ton lorry crane

Pallet truck

Wire/webbing sling

Obstacles

No obstacles on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, etc..)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lorry crane to park at designated location (back to back with container)

- Pull out machine case from container onto lorry chassis

- Hoist down case to the ground

- Uncrate case

- Hoist and detach machine from base

- Swing crane to loading area

- Insert pallet truck to machine base for the move

- Shift machine into production and position at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Remark:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175